AI-Based Wafer Cassette Inspection System

Project Proposal

Student Name: Anton Horvat

Company: Trymax Semiconductor Equipment B.V.

Department: Software / System Integration

Internship Period: February 9, 2025 - July 3, 2025 (20 weeks)

Date: February 2026

# Table of Contents

Table of Contents	1

1. What Are You Going to Do?	1

2. Why Are You Going to Do That?	1

2.1 Domain Understanding Research	1

Research Question	1

Research Methods	1

Domain Research Findings	1

Connection to Technical Approach	1

2.2 Analytic Approach	1

Problem Type Classification	1

Target Variable Definition	1

Algorithm Type Justification	1

2.3 Data Requirements	1

Objectives	1

Data Characteristics	1

Data Sources	1

Expected Good Indicators	1

2.4 Why This Project Delivers Real Business Value	1

Speed Improvement	1

Foundation for Smart Equipment	1

Trust Through Transparency	1

3. Who Is Involved (Stakeholders)?	1

4. When Is It Happening (Planning)?	1

Detailed Phase Breakdown	1

5. How Are You Going to Do It?	1

Following IBM Data Science Methodology	1

System Architecture	1

Iterative Process	1

Technology Stack	1

References	1

Semiconductor Manufacturing	1

Computer Vision and Transfer Learning	1

Explainable AI	1

Industrial AI and Inspection	1

(Right-click and select 'Update Field' in Word to populate)

# 1. What Are You Going to Do?

I will develop an AI-powered computer vision system that inspects wafer cassettes in real-time by analyzing camera images of cassette trays. The system will classify each individual slot in a cassette according to the industry-standard SlotOccupationState enum used by load port suppliers:

| Value | State | Description |
| --- | --- | --- |
| 0 | Undefined | Cannot determine slot state |
| 1 | Empty | No wafer present in slot |
| 2 | NotEmpty | Something detected but unclear classification |
| 3 | CorrectlyOccupied | Wafer properly placed in slot |
| 4 | DoubleSlotted | Two wafers stacked in one slot |
| 5 | CrossSlotted | Wafer extending across multiple slots |
| 6 | Reserved | Reserved for future use |

What makes my approach different: I will build a system that not only classifies slots but also explains its predictions visually using Explainable AI (Grad-CAM). The frontend will display a live camera feed showing the cassette inspection in real-time, with the AI predictions running side-by-side with the current mechanical mapper output. This allows engineers to validate the AI's accuracy against the existing system, building trust through transparent comparison. The end goal is a callable application that Trymax can integrate into their existing software stack, so that every time the application is called, it performs an AI inspection run and returns the SlotOccupationState array.

# 2. Why Are You Going to Do That?

Operational Motivation: The current wafer cassette mapping process takes 30 to 45 seconds per cassette. The mapper sits on the robot and mechanically inspects each slot sequentially. In semiconductor manufacturing, where throughput directly impacts revenue, this inspection bottleneck adds up significantly across hundreds of daily operations. An AI vision system performing the same inspection in under 1 second would represent a 30x to 45x speed improvement.

Strategic Motivation: Trymax Semiconductor Equipment B.V. is growing rapidly, with an installed base of 500+ process chambers targeting 1,000 by end of 2025. Customers in Asia (Taiwan, China, Korea) increasingly demand higher uptime, better process stability, and lower total cost of ownership. An AI-powered inspection system positions Trymax at the forefront of smart semiconductor equipment, differentiating their NEO series machines from competitors.

Technical Motivation: This project serves as a proof-of-concept to demonstrate that AI-based visual inspection is viable for Trymax equipment. If successful, it establishes the foundation for future AI applications across the product line and provides a reusable architecture (Python ML service + PHP backend) that can be extended to other inspection and monitoring tasks.

## 2.1 Domain Understanding Research

### Research Question

"How can a computer vision system reliably classify wafer cassette slot states from camera images with sufficient accuracy and speed to replace or augment the current mechanical mapper, while providing explainable predictions that semiconductor engineers can trust and validate?"

### Research Methods

On-Site Observation: Direct observation of NEO series machines, the cassette loading process, and the current mapper operation at Trymax's facility in Nijmegen. This includes understanding the physical cassette geometry, slot spacing, wafer types (6-inch and 8-inch), and cassette types (Type 1, 2, and 3).

Expert Interviews: Discussions with system integration engineers, software developers, and field service engineers at Trymax to understand common failure modes, the current mapping process, and requirements for a replacement system.

Technical Analysis: Study of the existing mapper system, SVCLog data format, sensor configurations (cassette present sensors, mapper sensors, up/down sensors, proximity sensors), and the SlotOccupationState enum standard used across load port suppliers.

Literature Review: Review of academic literature on computer vision for semiconductor manufacturing inspection, transfer learning with limited datasets, and Explainable AI techniques for industrial applications.

### Domain Research Findings

Key insights from domain research:

Machine Architecture:

- Trymax operates two main machine types: UV series and 2000 series, built differently but with similar sensor configurations

- The load port handles cassette types of different sizes, with sensors detecting whether it is a 6-inch or 8-inch wafer cassette

- The mapper is mounted on the robot arm and communicates via SVCLog

- Safety systems include light curtains (2000 series) and door sensors that halt robot operation

Current Inspection Process:

- The mapper performs sequential mechanical inspection: 30-45 seconds per cassette

- All data is logged in real-time via SVCLog on each machine's Windows-based IPC

- Data quality is very good and well-validated; nothing is deleted

- Mapper results are returned as a SlotOccupationState array (the target output for the AI system)

Common Failure Modes (from field service):

- Leaks, software bugs, process instability, generator failures

- Wear parts: lamps, chucks, air lines requiring regular replacement

- Maintenance is preventive: daily cleanup, monthly checkups, advanced sessions at 3/6/9/12 month intervals

- Temperature parameters typically 200 degrees; 300 degrees is critical (machine shutdown)

### Connection to Technical Approach

This domain research directly informs the technical approach in several ways. The fixed cassette geometry means slot positions are predictable, enabling a slot-segmentation approach rather than requiring complex object detection. The existing SlotOccupationState enum provides the exact classification target. The fact that mapper data is already logged via SVCLog means ground truth labels can be automatically generated for future model retraining by comparing AI predictions with mapper results. The variety of cassette types (different sizes, slot pitches) means the model must either handle multiple types or be configured per cassette type.

## 2.2 Analytic Approach

### Problem Type Classification

Multi-class Image Classification Problem. The system must classify each slot in a wafer cassette into one of the SlotOccupationState categories (primarily: Empty, CorrectlyOccupied, DoubleSlotted, CrossSlotted) based on visual features extracted from camera images. This is performed per-slot, meaning a single cassette image produces an array of classifications.

### Target Variable Definition

Target Variable: SlotOccupationState (Categorical, per slot)

- Empty (1): No wafer visible in slot position

- CorrectlyOccupied (3): Single wafer properly seated in slot

- DoubleSlotted (4): Two wafers stacked in one slot position

- CrossSlotted (5): Wafer extending across slot boundaries

- Undefined (0) / NotEmpty (2): Edge cases where classification is uncertain

This classification aligns with the universal industry standard used by all load port suppliers, ensuring compatibility with existing systems and workflows.

### Algorithm Type Justification

I will use a Convolutional Neural Network (CNN) approach with transfer learning:

- MobileNet / EfficientNet: Lightweight architectures designed for fast inference. Ideal for the <1 second requirement. Pretrained on ImageNet, providing strong feature extraction even with limited training data.

- ResNet (fallback): Deeper architecture if MobileNet accuracy is insufficient. Heavier but more powerful feature extraction.

- YOLO (alternative): If slot-segmentation proves difficult, object detection could locate and classify wafers directly in the full image.

Transfer learning is critical because the initial dataset will be small (iPhone prototype photos). Pretrained models bring knowledge of edges, shapes, and textures that transfer well to industrial inspection tasks.

## 2.3 Data Requirements

### Objectives

The data must enable the model to learn visual differences between empty slots, correctly placed wafers, double-slotted wafers, and cross-slotted wafers across varying lighting conditions and backgrounds.

### Data Characteristics

Required Features (Visual):

- Cassette slot region images (cropped from full cassette photo)

- Sufficient resolution to distinguish single vs double wafers

- Variation in lighting, angle, and background

- Multiple cassette types (6-inch, 8-inch; Type 1, 2, 3)

Data Quality Requirements:

- Labeled with verified SlotOccupationState per slot

- Minimum 50-100 images per class for initial prototype (more via augmentation)

- Diverse backgrounds to ensure model focuses on cassette, not environment

- Documented metadata: camera distance, angle, lighting conditions

### Data Sources

Primary Dataset: Self-collected iPhone photographs

- Captured at Trymax facility with varying distances and angles

- Folder structure serves as labeling: each folder name encodes the full cassette configuration using SlotOccupationState values

- Example: folder '13333333341333' means slot 1 empty, slots 2-9 correct, slot 10 double-slotted, etc.

Validation Dataset: Mapper output (SVCLog)

- The existing mechanical mapper provides ground truth SlotOccupationState arrays

- Can be used to automatically validate AI predictions during the side-by-side comparison phase

Data Augmentation: Synthetic expansion

- Random rotation, cropping, brightness/contrast variation

- Background replacement/blurring to force focus on cassette

- Horizontal flipping where physically valid

### Expected Good Indicators

Visual features with high predictive power:

- For Empty: Absence of horizontal wafer edge in slot region

- For CorrectlyOccupied: Single clean horizontal wafer edge at expected position

- For DoubleSlotted: Two visible edges or abnormal thickness in slot region

- For CrossSlotted: Wafer edge at diagonal angle or extending beyond slot boundaries

## 2.4 Why This Project Delivers Real Business Value

### Speed Improvement

- Current mapper: 30-45 seconds per cassette

- AI target: < 1 second per cassette

- That is a 30x to 45x throughput improvement for the inspection step

### Foundation for Smart Equipment

- First AI application at Trymax, establishing patterns for future ML projects

- Demonstrates that visual AI can augment or replace mechanical inspection

- Creates a reusable architecture: Python ML service callable from existing software

- Positions Trymax as an innovator in semiconductor equipment AI

### Trust Through Transparency

- Explainable AI (Grad-CAM) shows engineers exactly what the model sees

- Side-by-side comparison with mapper builds confidence in AI predictions

- Engineers can validate that the model focuses on wafer features, not artifacts

# 3. Who Is Involved (Stakeholders)?

Company Environment:

- Software Team (4 members): Primary collaborators. Will use and maintain the system after the internship. Critical for code review, integration guidance, and ensuring the solution fits existing architecture.

- System Integration (SIEE) Team: Responsible for machine integration. Key stakeholders for understanding how the AI system connects to load port operations and mapper commands.

- Process Engineers: Domain experts who understand what constitutes normal vs faulty wafer placement. Essential for validating model predictions and providing ground truth during development.

- Field Service Engineers: Understand common failure modes and customer pain points. Their input ensures the system addresses real operational needs.

- Company Supervisors / Mentors: Provide technical direction, access to machines and data, and evaluation of project progress.

Academic Environment:

- School Supervisor: Evaluates internship progress and academic deliverables.

This stakeholder structure ensures the AI system is built with direct input from the people who will use it, validate it, and integrate it into Trymax's product line.

# 4. When Is It Happening (Planning)?

20-Week Internship Timeline:

| Phase | Activities | Deliverable |
| --- | --- | --- |
| Week 1-3 | Onboarding, machine understanding, first iPhone photos, dataset v1, folder structure setup with SlotOccupationState naming | Dataset v1, data inventory document |
| Week 4-6 | Data preparation pipeline, image preprocessing, augmentation pipeline, ROI detection for cassette isolation, train/val/test split | Data pipeline, augmented dataset |
| Week 7-10 | Baseline model (transfer learning with MobileNet), slot classification experiments, per-slot evaluation, first Grad-CAM visualizations | Working baseline model, evaluation report |
| Week 11-14 | Model optimization, background robustness training, speed optimization (<1s target), advanced Grad-CAM analysis, mapper comparison | Optimized model, XAI report |
| Week 15-17 | FastAPI ML service, PHP backend integration, live camera feed frontend, side-by-side mapper comparison display | Working end-to-end prototype |
| Week 18-20 | Testing, documentation, knowledge transfer, final presentation, code handover | Final report, presentation, Git repo |

## Detailed Phase Breakdown

Weeks 1-3: Foundation and Data Collection

- Week 1: Onboarding, facility tour, machine understanding, meet all stakeholders

- Week 2: Begin iPhone photography sessions, establish folder naming convention using SlotOccupationState values, document camera distances and angles

- Week 3: Complete initial dataset, verify all labels, assess class distribution, identify which states need more samples

Weeks 4-6: Data Pipeline and Feature Engineering

- Week 4: Build Python data loading pipeline that reads folder structure as labels

- Week 5: Implement data augmentation (rotation, brightness, background variation), establish train/val/test split

- Week 6: ROI detection (cassette localization), slot segmentation based on cassette geometry, first transfer learning experiments

Weeks 7-10: Core Model Development

- Week 7: Train baseline CNN with MobileNet transfer learning

- Week 8: Evaluate per-slot accuracy, analyze confusion matrix, identify weak classes

- Week 9: Compare architectures (MobileNet vs EfficientNet vs ResNet), optimize for accuracy

- Week 10: First Grad-CAM implementation, validate model focuses on wafer features

Weeks 11-14: Optimization and Explainability

- Week 11: Train with diverse backgrounds, test robustness against environmental changes

- Week 12: Speed optimization (model pruning, quantization if needed) to hit <1 second target

- Week 13: Full XAI analysis with Grad-CAM, engineer validation sessions

- Week 14: Side-by-side comparison with mapper output, quantify agreement rate

Weeks 15-17: System Integration

- Week 15: Build FastAPI endpoint (/predict) that accepts images and returns SlotOccupationState array

- Week 16: PHP backend integration, database storage for predictions, image logging

- Week 17: Frontend with live camera feed, real-time predictions display, Grad-CAM overlay

Weeks 18-20: Documentation and Handover

- Week 18: Comprehensive testing, edge case analysis, performance documentation

- Week 19: Final report, technical documentation, installation guide

- Week 20: Presentation to Trymax team, code handover, knowledge transfer sessions

# 5. How Are You Going to Do It?

## Following IBM Data Science Methodology

Phase 1: Proposal (Business Understanding, Analytic Approach, Data Requirements)

- Business Understanding: Semiconductor cassette inspection is a bottleneck at 30-45 seconds. AI vision can reduce this to <1 second while maintaining accuracy. Trymax needs a proof-of-concept that can evolve into a production feature.

- Analytic Approach: Multi-class image classification per cassette slot, using the SlotOccupationState enum (6 states) as the target. CNN with transfer learning for the model; Grad-CAM for explainability.

- Data Requirements: Labeled cassette images with per-slot state annotations. iPhone photos as prototype data source, with mapper output as validation ground truth.

Phase 2: Provisioning (Data Collection, Data Understanding, Data Preparation)

- Data Collection: iPhone photography at Trymax facility. Folder-based labeling system where folder names encode the full cassette configuration (e.g., '13333333341333').

- Data Understanding: Analyze class distribution, identify underrepresented states, assess image quality across distances and lighting conditions.

- Data Preparation: Image resizing, ROI cropping, slot segmentation, data augmentation, train/val/test splitting. Build robust pipeline that can ingest new images as the dataset grows.

Phase 3: Predictions (Modeling, Evaluation)

- Modeling: Start with MobileNet baseline (Iteration Zero), then compare multiple CNN architectures. Two-stage pipeline: Stage 1 detects/crops the cassette, Stage 2 classifies each slot.

- Evaluation: Per-slot accuracy, per-class F1-scores, confusion matrix, inference time measurement. Side-by-side validation against mapper output. Grad-CAM analysis to ensure model learns meaningful features.

## System Architecture

The final system consists of three components working together:

1. Camera Input: Initially a laptop webcam (prototype), eventually a dedicated fixed camera mounted at the load port. Captures images of cassettes as they are loaded.

2. Python ML Service (FastAPI): Receives images via HTTP POST to /predict endpoint. Runs the two-stage pipeline (cassette detection + slot classification). Returns JSON with SlotOccupationState array and confidence scores. Also provides /explain endpoint for Grad-CAM visualizations.

3. PHP Backend: Receives camera images, forwards to Python ML service, stores predictions in database, serves the frontend. This is the callable application layer that Trymax's existing software can invoke. Each call triggers one complete AI inspection run.

This architecture means the AI inspection becomes a simple API call from Trymax's existing software: send an image, receive a SlotOccupationState array. The system is designed to be a drop-in addition, not a replacement of existing infrastructure.

## Iterative Process

After Iteration Zero proves feasibility with a basic MobileNet model on iPhone photos, each subsequent iteration adds complexity: more cassette types, background robustness, speed optimization, and finally live camera integration. The Grad-CAM analysis runs in parallel throughout, ensuring every model improvement is validated for explainability. If the slot-segmentation approach proves insufficient, the fallback is full-image object detection with YOLO.

## Technology Stack

| Category | Technology | Purpose |
| --- | --- | --- |
| ML Framework | Python + PyTorch | Model training and inference |
| Computer Vision | OpenCV | Image preprocessing, augmentation |
| Transfer Learning | MobileNet / EfficientNet | Pretrained feature extraction |
| Explainable AI | Grad-CAM | Visual explanation of predictions |
| ML API | FastAPI | Python inference endpoint |
| Backend | PHP | Application layer, data storage |
| Frontend | HTML/JS (live camera) | Real-time camera feed + results display |
| Version Control | Git | Code management and handover |

# References

## Semiconductor Manufacturing

Trymax Semiconductor Equipment B.V. (2026). NEO Series Product Documentation. Internal.

SEMI Standards. (2024). SEMI E84: Enhanced Carrier Handoff Parallel I/O Interface Standard.

## Computer Vision and Transfer Learning

Howard, A. G., et al. (2017). MobileNets: Efficient Convolutional Neural Networks for Mobile Vision Applications. arXiv:1704.04861.

Tan, M., & Le, Q. V. (2019). EfficientNet: Rethinking Model Scaling for Convolutional Neural Networks. ICML 2019.

He, K., et al. (2016). Deep Residual Learning for Image Recognition. CVPR 2016.

## Explainable AI

Selvaraju, R. R., et al. (2017). Grad-CAM: Visual Explanations from Deep Networks via Gradient-based Localization. ICCV 2017.

Adadi, A., & Berrada, M. (2018). Peeking Inside the Black-Box: A Survey on Explainable AI. IEEE Access, 6, 52138-52160.

European Parliament. (2024). Artificial Intelligence Act. Official Journal of the European Union.

## Industrial AI and Inspection

Weimer, D., Scholz-Reiter, B., & Shpitalni, M. (2016). Design of deep convolutional neural network architectures for automated feature extraction in industrial inspection. CIRP Annals, 65(1), 417-420.

Park, J. K., et al. (2016). Machine learning-based imaging system for surface defect inspection. International Journal of Precision Engineering and Manufacturing, 17(3), 303-311.